Task 2: Connecting and analysing the various results – Gate Transformer and Space-movement correlation analysis

Description

This exercise introduces the Gate Transformer tool and the QGIS spatial join functions to produce a simple space-movement correlation analysis. This exercise requires the pedestrian count dataset, the Axial map segment analysis dataset and the Catchment network dataset from the previous exercise.

- Prepare the project

  - Make the pedestrian_counts layer visible

  - Make the Axial_map_segment_analysis layer visible

  - Make the Catchment_network_line layer visible

- Run Gate Transformer – Resize

  - The aim of this step is to resize the movement gates so it has a constant length for visualisation

  - Start the Gate transformer tool by clicking on the gate transformer button or go to SSToolkit -> Gate Transformer

  - Select the pedestrian_counts layer

  - Click on the resize line radio button

  - Set the length to "25" metres

  - Click Transform and click Toggle Edit to save edits.

  - This resizes the movement gates to the same length of "25" metres

- Visualise the Pedestrian counts layer

  - Start the Attributes Explorer SST tool

  - Select the pedestrian_counts layer

  - Select the “PEDMOV” attribute

  - Select Symbology tab

  - Set colour range as “Classic”

  - Increase the line width to 0.5

  - Set Intervals to ‘Equal Intervals’

  - Click the Apply Symbology button

  - This is the standard display for the pedestrian movement data

  - You can add arrows by going to the ‘Style’ panel of the layer

  - Save an image of the map window

- Run Gate Transformer - Rotate

  - The aim of this step is to rotate the movement gates so it intersects with the segment layer

  - Start the Gate transformer tool by clicking on the gate transformer button or go to SSToolkit ->Gate Transformer

  - Select the pedestrian_counts layer

  - Click on the rotate radio button

  - Set the angle at “90deg”

  - Click Transform

  - The gates are now rotated by "90" degrees.

  - If the gates do not intersect with the segment layer, rotate again with a different angle until it intersects

  - Make sure the gates intersect the correct corresponding segment

- Pedestrian count gate and space syntax measures spatial join

  - The aim of this step is to join the pedestrian_count gate data layer and the Axial_map_segment_analysis layer

  - Go to the Vector menu at the top ->Data management tool -> Join attributes by location

  - Select the pedestrian_counts layer as Target vector layer

  - Select the Axial_map_segment_analysis layer as Join vector layer

  - Check the intersect button

  - Leave the other parameters as default

  - Press run to create a temporary joined layer

  - The temporary layer contains both the movement data and the space syntax measures where the two intersect

- Pedestrian count gate and catchment analysis spatial join

  - The aim of this step is to join the pedestrian counts gate data layer and the catchment analysis map

  - Go to the Vector menu at the top -> Data management tools -> Join attributes by location

  - Select the pedestrian_counts layer as Target vector layer

  - Select the catchment_network_line layer as Join vector layer

  - Check the intersect button

  - Leave the other parameters as default

  - Press run to create a temporary joined layer

  - The temporary layer shows both the movement data and the distance to tube stations where the two intersect

- Convert the PEDMOV column in the new joined layers from “string” to “integer” format

  - Double click on new Joined layer to get layer properties box

  - Select Fields tab

  - Click on the Field calculator button (abacus sign)

  - Check the Create New Field box

  - Type “PEDMOV_num” as Output field Name

  - Check that “Whole number (integer)” is selected in Output field type

  - In the Expression box, under row_number, select Conversions and double click on “to_int” from the drop-down menu

  - Now select Fields and Values and double click on “PEDMOV”

  - In the expression box to the left, add a “)” (close the bracket). The expression should now read: to_int(“PEDMOV”)

  - Click OK. You will see that a new “PEDMOV_Num” column has been added to your attribute table

  - Repeat step 7 for your other new Joined layer, so that you have PEDMOV_num interger columns in both Joined layers

  - Save both Joined layers as Shapefiles (otherwise they will be lost as Joined layers are temporary layers)

- Statistical scatterplot analysis of two variables

  - Start the Attributes Explorer SST tool

  - Select one of the newly saved Joined layer

  - Select Charts tab

  - Select Scatter plot button

  - Under numeric attributes or X-axis, select "INT" (or the name of a tube station)

  - Under Y-axis, select "PEDMOV_num"

  - The scatterplot shows the regression line, the equation of the line and the r2 (goodness of fit)